Setup NFS mount:

- sudo apt install network-manager

- touch /etc/NetworkManager/conf.d/100-disable-wifi-mac-randomization.conf

[connection]

wifi.mac-address-randomization=1

[device]

wifi.scan-rand-mac-address=no

- sudo systemctl enable NetworkManager.service

- sudo systemcrl start NetworkManager.service

- sudo mkdir -p /import/home

- /etc/fstab: 172.25.36.43:/export/home /import/home nfs defaults 0 0

- sudo mount -a

Add user:

- sudo adduser andy

- sudo adduser andy sudo

- sudo addgroup andy input

- Check 1001:1001

  - sudo usermod -u 1001 andy

  - sudo groupmod -u 1001 andy

  - sudo find / -group <old gid> -exec chgrp -h andy “{}” “;”

  - sudo find / -user <old uid> -exec chown -h andy “{}” “;”

- Add to ~andy/.bashrc

if [ -x /import/home/setupLinks.bash ]; then

/import/home/setupLinks.bash

fi

sudo raspi-config:

- Hostname: RaspPi4G

Preferences:

- Auto-Login: Disabled

- Interfaces: Enable VNC &SSH

Desktop Preferences:

- No Change

Citrix:

- https://medium.com/@swagattripathy/installing-citrix-workspace-app-on-raspberry-pi-4b-e14399d4b9f3

- https://www.citrix.com/downloads/workspace-app/

- cd ~/Downloads

- sudo dpkg -i icaclient_19.12.0.19_armhf.deb

- cd /opt/Citrix/ICAClient/keystore/cacerts

- sudo mkdir OLD; sudo mv * OLD

- for I in `ls /etc/ssl/certs`; do echo $I; sudo ln -s /etc/ssl/certs/$I $I; done

Other:

- sudo apt install gpsd gpsd-clients

- Download / install VNC Viewer

https://raspberrypi.org/documentation/hardware/raspberrypi/bcm2711_bootloader_config.md

https://zdnet.com/article/booting-my-raspberry-pi-4-from-a-usb-device

https://pimylifeup.com/raspberry-pi-boot-from-usb

https://raspberrypi.org/documentation/hardware/raspberrypi/bootmodes/msd.md

https://tomshardware.com/uk/how-to/boot-raspberry-pi-4-usb

Install moonlight:

- Update /etc/apt/sources.list

deb http://archive.itimmer.nl/raspbian/moonlight wheezy main

- Install moonlight-embedded

winetricks:

https://forums.flyer.co.uk/viewtopic.php?t=79588 x

Install winetricks

Set windows 10

winetricks dotnet30sp1

winetricks gdiplus

http://forums.skydemon.aero/Topic7466.aspx